IP Intercom Quick Manual Posted by edwin li on 24 November 2015 02:38 PM

Model:LCD1560BW, VTO2000A, POE1060A, DS-3503NO, PS2406,POE-TPE-S44 Title:IP Intercom Quick Manual Date: NOV 24,2015 Content: - Hardware Connection Diagram - Camera & Monitor Setup - Multiply Monitors Setup - Multiply Monitors Setup on Camera - Door Strike Setup Before Start: #1. A PC or Laptop connected to the local network and ready for use #2. All wires and connections are ready #3. Prepare IP address for device, using following information as the example: Room No.:101 Extension Room No.:101-1 Camera IP address: 192.168.0.149 Master monitor IP address: 192.168.0.150 Extension monitors IP address: 192.168.0.151 - Hardware Connection Diagram. 2.Camera & Monitor Setup Outdoor station Setup #1.Connect the outdoor station to the POE switch by using network cableand thenUsing IE browser to login to the outdoor station by using the information below: Default IP address: 192.168.1.110 User name: admin Password: admin. #2.Go to System Config-->Network Config to modify outdoor station IP address--> click OK. Indoor Monitor Setup #3. Connect the master monitor to POE switch -->setting-->Project Settings Password: 002236. #14.Open IE-->enter 192.168.0.149 to login to the outdoor station-->System Config-->Indoor Station Manager-->Add the information shown on the right side -->OK. Congratulations!! If customer needs to add more monitor, please continue... 3.Multiply Monitors Setup Note: Please make sure connect the Master monitor to outdoor station #1.Connect the extension monitor to POE switch-->setting-->Project Settings Password: 002236 --> Product info. 4. Multiply Monitors Setup on Camera #1.Open IE-->enter 192.168.0.149 to login to the outdoor station; Note: the device need to reboot after modify this configuration.. 5.Door Strike Connection Setup.