ImmunoBioScience Corp. (IBSC) DATA SHEET

Immuno Fluoro Mount™ (IFM) Mounting Medium

Catalog number: AR-6517-01 (30 ml) (This vial is closed with white flat cap and provided with extra WHITE Twist open/close cap for customer to replace it)

AR-6517-02 (100 ml)

Description: Immuno Fluoro Mount (IFM) is an aqueous non- permanent, low viscosity mounting medium for immunofluorescence. This unique formula prevents rapid photobleaching of FITC, Texas Red, AMCA, Cy2, Cy3, Cy5, Alexa fluoro 488, Alexa fluoro 594, Green fluorescent protein (GFP), tetramethyly rhodamine and Redox. The fluorescence is retained during storage at 4oC in the dark. Not recommended for phycoethyrin (PE), phycocyanin (PC) and allophycocyanin (APC).

This medium does not contain phenylenediamine, which destroys immunofluorescence of Cy dyes.

Refractive Index: 1.400 ± 0.002 (These numbers apply to these mounting mediums in solution. Refractive indexes change when the water solvent evaporates and mounting media dries on slides. We do not have the means to measure the refractive indexes of dry mounting mediums; however we expect the numbers to go higher when dried. The refractive index of water is 1.3330)

Intended Use: Immunofluorescence, confocal microscopy.

Reagent: Ready to use mounting medium for Immunofluorescence.

Storage: Stored at 2-8oC, DO NOT FREEZE.

Procedure:

- Bring the vial to room temperature.

- Rinse slide to be mounted with DISTILLED OR DEIONIZED WATER, touch the edges of slide on a paper towel to remove excess water. Place slides on a flat surface.

- Turn the vial upside down and open the dropper to remove any air bubbles.

Apply 2-3 drops of mounting medium and apply coverslip carefully without getting air

bubbles.

- The specimen is ready for visualization under a microscope.

- The coverslip may move, so it may be necessary to seal the edges of cover slip with nail polish, any organic medium or our Organo mounting medium (cat. # AR-6504).

- Store slide at 4ºC in the dark.

- Method for applying Coverslip: Put 1-2 drops of Mounting medium on the specimen. After 3-4 minutes apply coverslip carefully avoiding air bubbles. Put Kimwipes on the top of coverslip, press gently to remove excess mounting medium. With 200 micropipette add organic mounting medium to seal the edges. Incubate at 37Cfor one hour  in the dark to dry organic mounting medium.

Removal of Coverslip: Coverslip can be removed before sealing the edges. Soak slide in warm (37oC) distilled or deiononized water for several minutes. Carefully and slowly move the coverslip. Soak in water for an additional few minutes to remove coverslip. Rinse slide several times with warm water to remove all mounting medium. The slide can be remounted again.

--------------------------------------------------------------------------------------------------------------------

Limitation and warranty: Our warranty is limited to the actual price paid for the product. We are not liable for any property damage, personnel injury, time, effort or economic loss due to our product.

MSDS: This product contains 0.05 % sodium azide as a preservative, appropriate care should be taken in handling. National Institute of Occupational Safety and Health has warning that sodium azide can react with lead, copper, brass or solder in the pluming system and forms hydrazoic acid in acidic condition. Discard with copious amount of water. Avoid skin and eye contact with all laboratory products. Use appropriate lab. gear, lab coat , gloves and  safety glasses. Do not ingest any lab. products. This product is not approved for administration in human or animals.

For research use only; not for use in diagnostic procedures. FOR IN VITRO LABORATORY USE ONLY

“In vitro laboratory products for research”

Immuno Fluoro Mount is a trademark of ImmunoBioScience corp.

Phone:  + 1 425 367 4601; Fax:  + 1 425 367 4817; cell (mobile) phone:  + 1 425 314 0199

Marketing phone:  +1 650 343 IBSC (4272); e-mail:anitaIBSC@aol.com

Web site: www.immunobioscience.com; E-mail: baderbo@gmail.com  Country of origin: USA