### TWIN COMANCHE CHECK LIST

BEFORE STARTING ENGINES

- Preflight Complete.

- Seat Belts – ON

3.	Passengers Briefed

4.	Radio Master OFF

- All Electrical Switches Off

- Fuel – Mains

- Brakes TEST

- Gear Switch Down

- Alternate Air Off

- Cowl Flaps OPEN

## STARTING ENGINES

- Master switch – on.

- Magneto switches – on

- Generators - On

- Gear lights – check on

- Propeller Area clear

- Propeller control  – forward.

- Throttles/Mixture  – full open.

- Pumps On 3-5 seconds then off

- PUMPS OFF

- Mixtures Closed

- Throttles – ¼ inch Open  or micro switch click

- Engage Starter, ( Left engine) Advance Mixture when engine Fires (repeat for right engine)

- Throttle – Adjust (1000 rpm)

- Oil, fuel pressure – check.

- Lean for Taxi/number 4

##### BEFORE TAXI

- Anti – Collision Lights – On.

- Radios MASTER ON  RADIOS SET

- Clearances – ATIS, Clnc. Deliv., Grnd

- Transponder – Standby

- Nav. Lights Interior Lights  as Desired

- Brakes Test / Flaps UP

#### RUN UP

- Brakes – Set.

- Fuel – Mains

- Flight Controls Free and Correct.

- Flaps – Check all Positions.

- Oil Temp – In Green

- Run Up

- Mixture controls – forward Lean Above 4,000 feet

- Throttles controls – (1500 rpm)

- Props - exercise 3 times when cold.

- Throttle controls - (2000)

- Magnetos – check

Normal drop – 100 rpm.

Maximum drop – 175 rpm

Differential– 50 rpm

- Engine gauges – check.

- Vacuum – 4.8 to 5.1

(h)	Amps – Check

(i)	Throttles-idle/1000rpm RELEAN

- Flight Instruments – Set.

- Auto Pilot – Off

- Circuit Breakers – Check.

- Trim Tab – Take Off

- Radios Set Up  / Takeoff Clearance.

- Anti-Collision Lights – On.

- Cabin door and Window – Closed and locked.

- PRE TAKEOFF BREIFING

##### BEFORE TAKEOFF

- Transponder – On Alt

- Pumps – On

- Mixture Rich Below 4,000

- Props -  Forward

##### TAKE-OFF AND CLIMB  NORMAL TAKE OFF

- Throttles – forward

2.	Accelerate to 90 MPH (VR)

- Rotate @ 90 MPH (VR)

- Accelerate to Best Rate – 112 MPH

- Landing Gear Up when runway unusable

- Throttles 25 inches – Props 25 At 1000 AGL

Pumps off   Check fuel flow.

7. 	Enroute Climb 130 mph ( for cooling)

#### SHORT FIELD TAKE-OFF

- Flaps - bottom of white (flap gauge)

- Brakes Hold

- Throttles – forward

- Accelerate to 90 MPH

- Rotate @ 90 MPH

- Climb  Airspeed  90 mph

- Landing Gear Up

- Flaps Up

- Best Rate (112 MPH) when clear of obstacle

- At 1000’ AGL

Throttles 25 inches- Props 2500 rpm

Pumps OFF- check fuel flow

11.     Enroute Climb 130 MPH

CRUISE

- Power  for training  20”MP 2300 RPM

- Landing light off

- Cowl Flaps Closed  Check Temp 400 or less

- Mixture Lean Above 4.000 MSL Ft

#### BEFORE LANDING (Down Wind)

- 16-17 “ MP, 2300 RPM  120 MPH

- Gumps Check

- Gas on mains/pumps on

- Undercarriage  (gear down)

Below 150 MPH (check mirror)                    (check light)

- Mixture rich below 4000’

- Props 2500 (forward on Short Final)

###### Flaps as needed (below 125 mph)

#### NORMAL LANDING

1.	Base 110 MPH,

2.	Flaps as needed

- Final 90-95 MPH  (flat flair)

SHORT FIELD 90 MPH RED LINE

#### FULL FLAPS

#### AFTER LANDING

- Flaps retract.

- Cowl flaps Open/Lean Mixture Number4

- Landing lights / anti collision lights off.

- Fuel pumps off.

ENGINE SHUTDOWN

- Brakes apply and hold

- Radio master and standby off.

- Lights off

- Throttles 1200 RPM

- Mixtures idle cut off.

- Magnetos – off.

- Master Off.

ENGINE FAILURE (lower nose to Horizon)

- AIRSPEED  105mph (BLUE LINE)  min.

- BANK 3 to 5 into Good Engine (Step and Rock)

- Mixture Forward below 4000 ft

- Props Forward

- Throttles forward

- Flaps Up

- Gear Up

- (I) dentify  Step On Ball

- (V) erify    Retard Throttle on Dead Engine

- (F) ix First Look At Prop What is it doing? If wind milling proceed

- CHECK ALL  FUEL/AIR/MAG ITEMS or

- Mixture idle cut off / Prop Feather

- FUEL Pumps off

- MAGS Off on DEAD Engine

- Close Cowl On Dead Engine

- Open Cowl On Good Engine

- Trim Rudder as Needed

TO UNFEATHER PROP

- Throttle Open FULL

- *PROP  FORWARD*

- MIXTURE FORWARD

- NOSE SLIGHTY FORWARD

- Mags On

- Fuel Pump On

- Prime 3-5 seconds , FUEL PUMP OFF

- MIXTURE IDLE  CUT OFF

- Throttle 1/4 inch

- AIRSPEED 140 mph or until prop moves

- ENGAGE STARTER

- MIXTURE FORWARD TO MATCH OTHER ENGINE

- MP 12” UNTIL  ENGINE WARMS, WHEN WARM ADVANCE THROTTLE

- COWL FLAPS AS NEEDED

IF UNABLE TO START IN 10 SECONDS, LET STARTER COOL FOR 1 MIN. THEN TRY AGAIN..

* TURN FUEL PUMP ON “ re-prime as above