Company Name Forklift Safety Quiz #2

| Instructor: |  | Name: |  |
| --- | --- | --- | --- |
| Class: |  | Date: |  |
| Time: |  | Results: |  |

Instructions

Read each question carefully and mark the correct answer.  If you have any questions, raise your hand and the instructor will assist you.

- An  employee must be at least ________________ years old to operate a forklift.

- What does the E type designation on a forklift mean?

  - It can be used indoors or outdoors.

  - It is electrically powered.

  - It is diesel fuel powered.

  - It is safe to use in hazardous atmospheres.

- When the center of gravity is inside the stability triangle, the load is stable.

True	False

- Forklifts must be certified by ANSI and marked as such.		True	False

- Forklift modifications must be:

  - Approved in writing by the manufacturer.

  - Just approved by management.

  - Made by the operator as long as they do not affect vehicle stability.

  - Made by a competent person.

- The trucks resistance to turning over sideways is known as its:

  - Center of gravity.

  - Turn resistance.

  - Lateral stability.

  - Dynamic stability.

- Forklifts must have overhead guards to protect operators from falling objects.

True	False

- Operators must wear seatbelts.		True	False

- What should you do if you are seated in a forklift and it tips over?

- Which direction should you face when entering or exiting a powered industrial truck?

- Forklifts may not be parked within _________ feet of railroad tracks.

- Forklifts must maintain at least ___________  vehicle lengths from the forklift ahead.

- Why is it important to make slow and controlled turns?

- What special precautions must be taken for loading or unloading highway trucks?

- Fuel tanks may be filled while the forklift engine is running.		True	False