STANDARD OPERATING PROCEDURE – HURRICANE PREPARATION

(Atlantic Hurricane Season officially starts on June 1 and ends on November 30)

During the month of May

- Check telephone tree of people responsible for the fish facility

- Check hurricane preparedness kit. The kit should include

  - Flashlights in working conditions, with batteries

  - Plastic sheets sufficient to cover fish racks

  - Disposable camera

  - Watertight container

  - Duct tape

- Check fish facility toolbox. The toolbox should include

  - Screwdrivers

  - Pliers

  - Sealant (approved for use for aquaria)

  - Lubricant (approved for use in aquaria)

  - Teflon tape

- Replace batteries on monitoring systems

Five days before an imminent storm

- Request 10 sandbags

- Back up computers to external hard drive

- For irreplaceable fish lines:

  - Cross and ship embryos via Federal Express to laboratories in other cities.

  - Divide population of adult fish and bring some to Medical School.

Two days before imminent storm

- Drape plastic sheets over fish racks and equipment (e.g., injectors)

Before leaving fish facility before storm

- Water shutdown

  - Unplug water exchangers in both system A and system B

  - RO water reservoir in mechanical room

    - Fill reservoir tank with RO water

    - Close recirculating valve

  - RO unit

    - Deactivate Amega monitoring system

    - Close water intake and outtake

    - Turn RO system off

  - Water Softener

    - Turn water softener off

- Set up A/C unit in mechanical room, leave door ajar

- Put sandbags against doors to minimize flooding

- Put records, external hard drive, toolbox, flashlight, disposable cameras in waterproof container near entrance.

- Final inspection