WASTE TREATMENT SYSTEM

NOTICE TO HOMEOWNER - The wastewater that is produced by our residences is treated by an on-site advanced wastewater treatment system.   The system will work properly provided we do not place harmful substances in the system.  Therefore, certain standards must be followed in order to receive optimal performance from our system. You may be interested to know that most of these same standards are also important to follow even for those who are connected to a municipal or city sewer treatment system or conventional septic systems. Please refer to the list below for important information on how to help keep our treatment system performing as it should.

1.       DISINFECTANTS OR BLEACHES - Use in accordance with manufacturers recommendations and sparingly. Quaternary ammonia sanitizers (found primarily in commercial settings) or pine oil cleaners should not be used.

2.       MEDICATIONS - Normal use of over the counter medications should not affect the system. However, strong antibiotics or chemotherapy drugs have been known to severely disrupt the treatment process.  Please notify the Board of this kind of issue so they may be aware of the reason for the system upset.

3.       DETERGENTS - Should be low-suds, biodegradable, and low phosphate.

4.       PAPER PRODUCTS - Use white toilet paper products. Some natural bacteria do not eat color dyes in paper and therefore do not breakdown colored paper.  Non-bleached paper (brown in color) takes a long time to breakdown due to the increased level of wood pulp.

5.       NO TOILET BOWL TABLETS SHOULD BE USED.

6.       NO ANIMAL FATS, SUCH AS BACON GREASE, LARD, OR ANY OTHER OILS SHOULD GO INTO THE SYSTEM - Normal cleaning of pots and pans is acceptable.

7.       NO DRAIN CLEANERS SHOULD BE USED - Vinegar and baking soda is good substitute. As a last resort chemical drain cleaners can be used, but use sparingly because they are toxic to the bacteria in the system.

8.       NO LIQUID FABRIC SOFTENERS SHOULD BE USED - These products typically contain quaternary ammonia which is toxic to the bacteria. Please use dryer sheets.

9.       NO HARSH CHEMICALS OR TOXINS SHOULD BE PUT INTO THE SYSTEM – Such as floor stripping waste, household paints, solvents, thinners, caustic cleaners, pesticides, herbicides etc.

10.   OBJECTS NOT TO BE PUT INTO THE SYSTEM - Disposable diapers, cat litter, sanitary napkins, bandages, cigarette butts, automotive fluids, rags, string, sticks, mud, condoms, plastics, metal objects, paper towels, corn cobs, animal bones, melon rinds, coffee grounds, home brewery waste, egg shells, old medications, baby wipes, hand wipes or fabric softener tissues.

11.   LAUNDRY WASH LOADING - It is recommended that wash loads be spread throughout the week.

12.   SEPTIC TANK ADDITIVES, ENZYMES, AND BACTERIA - Unnecessary with advanced treatment systems.