# MMB pin identification

Here is an explanation of the individual pins of the individual rows in detail.
First so that we talk about the same pins and row locations the vertical rows will 
be numbered from left to right from one (1) to seven (7) and the pins per row will be 
numbered from top to bottom as ten (10 Top) twenty (20 Middle) and thirty 
(30 Bottom).

All rows from 1 to 7 have their 30 pins tied to ground that comes in from the
RX AUX output.

Rows 4,5,6 and 7 have their 20 pins tied to +5V that comes from the RX AUX
output.

Row #1 Pin 10 - comes from the RX Aileron Signal Output pin
Row #1 Pin 20 - comes from the RX Elevator Signal Output pin
Row #1 Pin 30 - is Ground

Row #2 Pin 10 - goes to the T1000FC Aileron Signal Input pin
Row #2 Pin 20 - goes to the T1000FC Elevator Signal Input pin
Row #2 Pin 30 - is Ground

Row #3 Pin 10 - comes from the T1000FC Aileron Signal Output pin
Row #3 Pin 20 - comes from the T1000FC Elevator Signal Output pin
Row #3 Pin 30 - is Ground

Row #4 Pin 10 - goes to the Aileron Servo Signal pin
Row #4 Pin 20 - is 5Volts
Row #4 Pin 30 - is Ground

Row #5 Pin 10 - goes to the Elevator Servo Signal pin
Row #5 Pin 20 - is 5Volts
Row #5 Pin 30 - is Ground

Row #6 Pin 10 - goes to the T1000FC Auto Pilot Signal pin
Row #6 Pin 20 - is 5Volts
Row #6 Pin 30 - is Ground

Row #7 Pin 10 - comes from the RX Aux Signal pin
Row #7 Pin 20 - is 5Volts (supplied by the RX)
Row #7 Pin 30 - is Ground (supplied by the RX)

BTW the first three rows #1, #2, #3 have special servo leads 
that are supplied and look like the the pic below. The Connector
on the left with the 3 wires plugs into the MMB PCB.

Rows #4 and #5 use the lead attached to the Ail and Ele Servo.
Rows #6 and #7 use standard Male to Male Servo extensions.