SAFETY DATA SHEET

Section 1 IDENTIFICATION

Product Name: RD-61 Citra Plus Chemical Name/Synonyms: Multi- Purpose Citrus Cleaner Manufacturer: ICE Companies, Inc. P.O. Box 66 Date Revised: 08/03/2015_ Wilmington, NC 28412 Emergency Numbers: (910) 791-1970 Medical: (800) 791-9595 Transportation: (800) 424-9300 Recommended Use: Multi-Purpose Cleaner

Section 2 HAZARD IDENTIFICATION

(1.) Hazard Classification: Non-Hazardous Cleaner (2.) Chemical Formula: Proprietary______ (3.) NFPA & HMIS acute Hazard Rating: Heath: 1 _Flammability: 0_Reactivity: 0__ (4.) Preparer Name: Larry Gay_________ (5.) Personal Protective Equipment: Safety glasses and gloves__

Section 3 CHEMICAL COMPOSITION

note: list all CERCLA hazardous substances at 1% of greater and carcinogens at 0.1% or greater Ingredient:________________________________ OSHA/PEL AGGIH/TLV Other Exposure Limits CAS Number D-Limonene (Citrus Oil) NE NE 5989-27-5 2 Butoxyethanol – Butyl Cellosolve 240mg/m3 97mg/m3 20 PPM 111-76-2 Sodium Metasilcate NE NE 6814-92-0 Surfactant – Proprietary Blend NE NE *The specific chemical percentage has been withheld as a trade secret

Section 4 FIRST-AID MEASURES

(1.) Primary route(s) of entry into body: (2.) Eyes: X__ (3.) Skin:_ X_ (4.) Inhalation: __ (5.) Ingestion:_X__ (6.) Acute effects: Eyes: tissue irritation, redness Skin: tissue irritation, redness Inhalation: NA Ingestion: possible nausea Fiirst-Aid Instructions: - Eyes: remove contact lenses at once, flush with cool water for 30 minutes and call physician - Skin: wash with cold water for 15 minutes, neutralize with vinegar or citrus juice and call physician - Inhalation: N/A - Ingestion: Do not induce vomiting. Give large amounts of water to victim and call physician

Section 5 FIRE FIGHTING MEASURES

(1.) Extinguisher media: water, foam, or CO2 (2.) Hazardous combustion byproducts: None (3.) Recommended fire-fighting procedures: usual procedures__ (4.) Unusual fire explosion hazards: none__

Section 6 ACCIDENTAL RELEASE MEASURES

- Spill or leak procedures: Large: dike and dilute with water and neutralize with weak mineral acid; small: absorb on oil dry and place in approved container for disposal; neutralize area with small amount of weak mineral acid solution - Waste disposal method: dispose of in accordance with Federal, State, and Local regulations - EPA appropriate waste classification: n/a__ (4.) RCRA appropriate characteristic waste: no__ (5.)RCRA appropriate listed waste: no If so, EPA Hazardous Waste # ____________ (6.) Procedure for handling empty containers: do not reuse, return to licensed drum reconditioner (7.) Environmental toxicity data: Biodegradable_ (8.) Is material classified under the CLEAN WATER ACT (USA) or appropriate water regulation as a: Toxic pollutant (section 307): No_ Hazardous substance (section 311): YES_ if yes, reportable quantity (R.Q): none___ Oil (section 311): No_ (9.) Is material classified under the CLEAN AIR ACT (USA) or appropriate CLEAN AIR regulation as a: Hazardous air pollutant (section 12): No_ (10.) Comments: None_

Section 7 HANDLING & STORAGE

- Storage: store in cool, dry location away from strong acids, keep container tightly closed - Handling: wear chemical resistant gloves and goggles - Precautions: do not breath vapors or mist, do not wear contact lenses - Unusual Precautions: NONE

Section 8 EXPOSURE CONTROLS/PERSONAL PROTECTION

- Ventilation:___ (2.) Local exhaust: X_ (3.) General exhaust: ___ (4.) None required:____ (5.) Personal protective equipment: Ventilation: local exhaust to maintain the TLV Respirator type: NIOSH or MSHA Approved Respirator Gloves: Neoprene or PVC (6.) Eye protection: safety goggles___

Section 9 PHYSICAL AND CHEMICAL PROPERTIES

- Appearance: White Milky Liquid / Orange _____ (2.) Odor: Orange Scent____(3.) Physical State: Liquid_____________ - Boiling Point: 212°F______ (5.) Freeze Point: n/a_ (6.) Specific gravity: 1.04__ ) - Water Solubility: Complete_______(8.) Vapor Pressure: n/a________ (9.) Vapor Density: n/a_____________ (10.) Evaporation rate: ≥1 (11.) VOC Less Exempt (Theoretical): 0.0lbs./gal (12.) VOC as Packaged (Theoretical): 0.0lbs./gal *information not provided is either irrelevant or unavailable

Section 10 STABILITY AND REACTIVITY DATA

Reactivity: This product may react with acids Chemical Stability: This chemical is stable under normal ambient temperature and conditions while in storage and being handled Other: - Thermal Stability: STABLE___ If yes, conditions to avoid: irregular conditions___ - Hazardous decomposition products: NONE________ - Hazardous polymerization: May occur X___ Will not occur: ______ - Materials to avoid: Strong acids _____________________________________________ - Corrosive action on materials: ___________________________

Section 11 TOXICOLOGICAL INFORMATION

Summary of health effects: Mildly hazardous to all routes of body, seek physician; irritant to skin and eyes (1.) Primary route(s) of entry into body: (2.) Eyes: X__ (3.) Skin:_ X_ (4.) Inhalation: _ (5.) Ingestion:_X_ (6.) Acute effects: Eyes: tissue irritation, redness Skin: tissue irritation, redness Inhalation: N/A Ingestion: possible nausea *None of the components of this product are recognized as carcinogenic

Section 16 Other Information

SDS was prepared on 8/3/2015. Changes have been made to comply with Appendix D of 29 CFR 1910.1200.

**Sections 12-15 are non-mandatory and have been omitted