Dry Cargo Container. 20' Steel Dry Cargo Container. Exterior. Length; Width; Height. 20'-0"; 8'-0"; 8'-6". 6.058 m; 2.438 m; 2.591 m. Interior. Length; Width; Height. 19'-4 13/16"; 7'-8 19/32"; 7'-9 57/64". 5.898 m; 2.352 m; 2.385 m. Weight; Door Opening. MGW; TARE; NET; Width; Height. 52,910 lb; 5,140 lb; 47,770 lb; 7'-8 1/8"; 7'-5 3/4". 67,200 lb; 5,290 lb; 61,910 lb; 2.343 m; 2.280 m. 24,000 kg; 2,330 kg; 21,670 kg; CU.M; CU.FT. 30,480 kg; 2,400 kg; 28,080 kg; 33.1; 1,169. Purpose. 1.Captioned units(MGW 30,480 KG) can be coordinated from EMCU 3204073 and EISU 3568118. 2.Used for all kinds of general cargo... 40' Steel Dry Cargo Container. Exterior. Length; Width; Height. 40'-0"; 8'-0"; 8'-6". 12.192 m; 2.438 m; 2.591 m. Interior. Length; Width; Height. 39'-5 45/64"; 7'-8 19/32"; 7'-9 57/64". 12.032 m; 2.352 m; 2.385 m. Weight; Door Opening. MGW; TARE; NET; Width; Height. 67,200 lb; 8,820 lb; 58,380 lb; 7'-8 1/8"; 7'-5 3/4". 2.343 m; 2.280 m. 30,480 kg; 4,000 kg; 26,480 kg; CU.M; CU.FT. 67.5; 2,385. Purpose. Used for all kinds of general cargo... 40' Hi-Cube Steel Dry Cargo Container. Exterior. Length; Width; Height. 40'-0"; 8'-0"; 9'-6". 12.192 m; 2.438 m; 2.896 m. Interior. Length; Width; Height. 39'-5 45/64"; 7'-8 19/32"; 8'-9 15/16". 12.032 m; 2.352 m; 2.69 m. Weight; Door Opening. MGW; TARE; NET; Width; Height. 67,200 lb; 9,260 lb; 57,940 lb; 7'-8 1/8"; 8'-5 49/64". 2.343 m; 2.585 m. 30,480 kg; 4,200 kg; 26,280 kg; CU.M; CU.FT. 76.2; 2,690. Purpose. Used for all kinds of general cargo... 45' Hi-Cube Steel Dry Cargo Container. Exterior. Length; Width; Height. 45'-0"; 8'-0"; 9'-6". 13.716 m; 2.438 m; 2.896 m. Interior. Length; Width; Height. 44'-5 7/10''; 7'-8 19/32''; 8'-10 17/64''. 13.556 m; 2.352 m; 2.698 m. Weight; Door Opening. MGW; TARE; NET; Width; Height. 67,200 lb; 10,580 lb; 56,620 lb; 7'-8 1/8"; 8'-5 3/4". 2.340 m; 2.585 m. 30,480 kg; 4,800 kg; 25,680 kg; CU.M; CU.FT. 86.1; 3,040. Purpose. Used for all kinds of general cargo.. Exterior. Length; Width; Height. 45'-0"; 8'-0"; 9'-6". 13.716 m; 2.438 m; 2.896 m. Interior. Length; Width; Height. 44'-5 7/10''; 7'-8 19/32''; 8'-10 17/64''. 13.556 m; 2.352 m; 2.698 m. Weight; Door Opening. MGW; TARE; NET; Width; Height. 67,200 lb; 10,858 lb; 56,342 lb; 7'-8 1/8"; 8'-5 3/4". 2.340 m; 2.585 m. 30,480 kg; 4,870 kg; 25,610 kg; CU.M; CU.FT. 86; 3,040. Purpose. Captioned units can be coordinated from EISU 804001~EISU 804500 and EMCU 801001~EMCU 80250.. Exterior. Length; Width; Height. 45'-0"; 8'-0"; 9'-6". 13.716 m; 2.438 m; 2.896 m. Interior. Length; Width; Height. 44'-5 7/10''; 7'-8 19/32''; 8'-10 17/64''. 13.556 m; 2.352 m; 2.698 m. Weight; Door Opening. MGW; TARE; NET; Width; Height. 71,650 lb; 10,360 lb; 61,290 lb; 7'-8"; 8'-6". 2.340 m; 2.585 m. 32,500 kg; 4,700 kg; 27,800 kg; CU.M; CU.FT. 86; 3,040. Purpose. Captioned units can be coordinated from EISU 802001~EISU 804000....

Refrigerated Container. 20' M.G.S.S. Refrigerated Container. Exterior. Length; Width; Height. 20'-0"; 8'-0"; 8'-6". 6.058 m; 2.438 m; 2.591 m. Interior. Length; Width; Height. 18'- 17/32"; 7'-6 15/32"; 7'-5 39/54". 5.500 m; 2.298 m; 2.276 m. Nominal; Weight. Cubic Capacity; MGW; TARE. 1,016 cu.ft.; 67,180 lb; 6,700 lb. 28.8 cu.m.; 30,480 kg; 3,020 kg. Cooling capacity; Air exchange rate; Type; Temp. control precision. w(kcal)/C(F); CFM; cu.m/hr. 11,000(9,460)/1.7(35) 6,280(5,400)/-18(0); 0-106; 0-180; MHI CPE14-2BAIIIEU; +-0.25C. 10,550(9,073)/2(35) 6,150(5,289)/-18(0); 0-142; 0-240; CARRIER 69NT40-551-501; +-0.25C.. 20' Aluminum Refrigerated Container. Exterior. Length; Width; Height. 20'-0"; 8'-0"; 8'-6". 6.058 m; 2.438 m; 2.591 m. Interior. Length; Width; Height. 17'-0"; 7'-6"; 7'-3". 5.480 m; 2.286 m; 2.235 m. Nominal; Weight. Cubic Capacity; MGW; TARE. 989 cu.ft.; 52,800 lb; 6,170 lb. 28 cu.m.; 24,000 kg; 2,800 kg. Cooling capacity; Air exchange rate; Type; Temp. control precision. w(kcal)/C(F); CFM; cu.m/hr. 11,000(9,460)/1.7(35) 6,280(5,400)/-18(0); 0-106; 0-180; MHI CPE14-2BAIIIEU; +-0.25C.. 40' M.G.S.S. Hi-Cube Refrigerated Container. Exterior. Length; Width; Height. 40'-0"; 8'-0"; 9'-6". 12.192 m; 2.438 m; 2.896 m. Interior. Length; Width; Height. 37'-11 55/64"; 7'-6 15/32"; 8'-4 5/32". 11.585 m; 2.290 m; 2.544 m. Nominal; Weight. Cubic Capacity; MGW; TARE. 2,384 cu.ft.; 74,960 lb; 10,490 lb. 67.5 cu.m.; 34,000 kg; 4,760 kg. Cooling capacity; Air exchange rate; Type; Temp. control precision. w(kcal)/C(F); CFM; cu.m/hr. 11,000(9,460)/1.7(35) 6,280(5,400)/-18(0); 0-106; 0-180; MHI CPE16-2BAIIIEV; +-0.25C. 10,550(9,073)/2(35) 6,150(5,289)/-18(0); 0-142; 0-240; CARRIER 69NT40-551-501; +-0.25C.. 40' Hi-Cube Aluminum Refrigerated Container. Exterior. Length; Width; Height. 40'-0"; 8'-0"; 9'-6". 12.192 m; 2.438 m; 2.896 m. Interior. Length; Width; Height. 37'-11"; 7'-6"; 8'-2". 11.563 m; 2.286 m; 2.507 m. Nominal; Weight. Cubic Capacity; MGW; TARE. 2,340 cu.ft.; 67,200 lb; 9,480 lb. 66.27 cu.m.; 30,480 kg; 4,300 kg. Cooling capacity; Air exchange rate; Type; Temp. control precision. CFM; cu.m/hr; w(kcal)/C(F). 11,000(9,460)/1.7(35) 6,280(5,400)/-18(0); 0-106; 0-180; MHI CPE14-2BAIIIA/ES; +-0.25C...

Special Container. 20' Full Height Open Top Container. Exterior. Length; Width; Height. 20'-0"; 8'-0"; 8'-6". 6.058 m; 2.438 m; 2.591 m. Interior. Length; Width; Height. 19'-4"; 7'-8 1/2"; 7'-8 1/8". 5.898 m; 2.352 m; 2.342 m. Weight. MGW; TARE; NET. 44,800 lb; 4,850 lb; 39,950 lb. 20,320 kg; 2,200 kg; 18,120 kg. CU.M; CU.FT. 32.5; 1,148. Purpose. Captioned units can be coordinated from UGMU 490001~UGMU 490150, EMCU 460261~EMUC 460520 and EISU 460001~EISU 460400... 20 ' Flat Rack Container with Four Freestanding Posts. Exterior. Length; Width; Height. 20'-0"; 8'-0"; 8'-6". 6.058 m; 2.438 m; 2.591 m. Interior. Length; Width; Height. 18'-6 7/16"; 6'-7 59/64"; 6'-9 39/64". 5.650 m; 2.030 m; 2.073 m. Weight. MGW; TARE; NET. 66,140 lb; 6,150 lb; 59,990 lb. 30,000 kg; 2,790 kg; 27,210 kg. Purpose. Captioned units can be coordinated from EMCU 760321~EMCU 760520... 20' Flat Rack Container with Collapsible End. Exterior. Length; Width; Height. 20'-0"; 8'-0"; 8'-6". 6.058 m; 2.438 m; 2.591 m. Interior. Length; Width; Height. 18'-5 62/64"; 7'-3 46/64"; 7'-3 59/64". 5.638 m; 2.228 m; 2.233 m. Weight. MGW; TARE; NET. 74,950 lb; 6,370 lb; 68,580 lb. 34,000 kg; 2,890 kg; 31,110 kg. Purpose. Captioned units can be coordinated from EISU 750000 to EISU 750300... 20' Fuel Tank Container. Exterior. Length; Width; Height. 20'-0"; 8'-0"; 8'-6". 6.058 m; 2.438 m; 2.591 m. Weight. MGW; TARE; NET. 36,000 kg; 3,385 kg; 32,615 kg. Capacity; Design Temp.. 24,000 litres; -20°C to 130°C. Purpose. Captioned units can be coordinated from EISU 8010010 to EISU 8010453, and are need for power pack generators equipped on group vessels, not for export or import purpose.... 20' Power Pack Generator Container. Exterior. Length; Width; Height. 20'-0"; 8'-0"; 8'-6". 6.058 m; 2.438 m; 2.591 m. Interior. Length; Width; Height. 19'-4 13/16"; 7'-8 19/32"; 7'-9 57/64". 5.898 m; 2.352 m; 2.385 m. Weight. MGW; TARE; MACHINE. 8,500 kg; 2,400 kg; 6,100 kg. Power Supply; Socket. 450 kw; 40. Purpose. Captioned units can be coordinated from EMCU 0000013 to EMCU 0000409... 40' Flat Rack Container with Four Freestanding Posts. Exterior. Length; Width; Height. 40'-0"; 8'-0"; 8'-6". 12.192 m; 2.438 m; 2.591 m. Interior. Length; Width; Height. 38'-7 15/16"; 6'-7 59/64"; 6'-4 1/2". 11.784 m; 2.030 m; 1.943 m. Weight. MGW; TARE; NET. 99,210 lb; 11,908 lb; 87,302 lb. 45,000 kg; 5,400 kg; 39,600 kg. Purpose. Captioned units can be coordinated from UGMU 707001~UGMU 707050; EMCU 700251~EMCU 700450... 40' Flat Rack Container with Collapsible End. Exterior. Length; Width; Height. 40'-0"; 8'-0"; 8'-6". 12.192 m; 2.438 m; 2.591 m. Interior. Length; Width; Height. 38'-7 39/64"; 7'-3 46/64"; 6'-4 61/64". 11.776 m; 2.228 m; 1.955 m. Weight. MGW; TARE; NET. 99,210 lb; 10,140 lb; 89,070 lb. 45,000 kg; 4,600 kg; 40,400 kg. Purpose. Captioned units can be coordinated from EISU 710000~EISU 710500, EMCU 700451~EMCU 700650.. 40' Flat Rack Hi-Cube Container. Exterior. Length; Width; Height. 40'-0"; 8'-0"; 9'-6". 12.192 m; 2.438 m; 2.896 m. Interior. Length; Width; Height. 38'-2 3/4"; 7'-9 7/16"; 7'-5 1/8". 11.652 m; 2.374 m; 2.264 m. Weight. MGW; TARE; NET. 115,740 lb; 11,460 lb; 104,280 lb. 52,500 kg; 5,200 kg; 47,300 kg. Purpose. Captioned units can be coordinated from EMCU700681-700780.. 40' Full Height Open Top Container. Exterior. Length; Width; Height. 40'-0"; 8'-0"; 8'-6". 12.192 m; 2.438 m; 2.591 m. Interior. Length; Width; Height. 39'-5"; 7'-8 1/2"; 7'-8 1/8". 12.034 m; 2.352 m; 2.330 m. Weight. MGW; TARE; NET. 67,200 lb; 9,040 lb; 58,160 lb. 30,480 kg; 4,100 kg; 26,380 kg. CU.M; CU.FT. 65.9; 2,327. Purpose. Captioned units can be coordinated from EMCU 400362~EMCU 400810, UGMU 407001~UGMU 407050 and EISU 400001~EISU 400600... 40' Hi-Cube Open Top Container. Exterior. Length; Width; Height. 40'-0"; 8'-0"; 9'-6". 12.192 m; 2.438 m; 2.896 m. Interior. Length; Width; Height. 39'-6"; 7'-9"; 8'-8 7/16". 12.032 m; 2.352 m; 2.653 m. Weight. MGW; TARE; NET. 67,200 lb; 9,390 lb; 57,810 lb. 30,480 kg; 4,260 kg; 26,220 kg. CU.M; CU.FT. 75.1; 2,650. Purpose. Captioned units can be coordinated from EMCU400911-401010.. 40' Hi-Cube Hanger Container. Exterior. Length; Width; Height. 40'-0"; 8'-0"; 9'-6". 12.19 m; 2.44 m; 2.90 m. Interior. Length; Width; Height. 39'-5 45/64"; 7'-8 19/32"; 8'-9 15/16". 12.03 m; 2.35 m; 2.69 m. Weight; Door Opening. MGW; TARE; NET; Width; Height. 67,200 lb; 9,172 lb; 58,028 lb; 7'-8 1/8"; 8'-5 49/64". 30,480 kg; 4,200 kg; 26,280 kg; 2.343 m; 2.585 m. Purpose. Used for all kinds of garment....