SAFETY DATA SHEET

Section 1 IDENTIFICATION

Product Name: AB-1000 Chemical Name/Synonyms: Aluminum Brightner Manufacturer: ICE Companies, Inc. P.O. Box 66 Date Revised: 06/08/2015_ Wilmington, NC 28412 Emergency Numbers: (910) 791-1970 Medical: (800) 791-9595 Transportation: (800) 424-9300 Recommended Use: Aluminum Cleaner

Section 2 HAZARD IDENTIFICATION

(1.) Hazard Classification: Class 8 - Corrosive (2.) Chemical Formula: Proprietary______ (3.) NFPA & HMIS acute Hazard Rating: Heath: 3 _Flammability: 0_Reactivity: 2__ (4.) Preparer Name: Larry Gay_________ (5.) Personal Protective Equipment: Safety glasses and gloves, Face Shield, no contact lenses

Section 3 CHEMICAL COMPOSITION

note: list all CERCLA hazardous substances at 1% of greater and carcinogens at 0.1% or greater Ingredient:________________________________ OSHA/PEL AGGIH/TLV Other Exposure Limits CAS Number Phosphoric Acid 7664-38-2 Glycol Ether EB 50 PPM 111-76-2 Hydrofluoric Acid – 3% to 8% by Volume 3 PPM Air 2.5 mg/m³ 7664-39-3 *The specific chemical percentage has been withheld as a trade secret

Section 4 FIRST-AID MEASURES

(1.) Primary route(s) of entry into body: (2.) Eyes: X__ (3.) Skin:_ X_ (4.) Inhalation: X __ (5.) Ingestion:_X__ (6.) Acute effects: Eyes: tissue irritation, redness, blurred Vision Skin: tissue irritation, redness Inhalation: Shortness of breath Ingestion: Gastro-intestinal distress Fiirst-Aid Instructions: - Eyes: flush with cool water for 30 minutes and call physician - Skin: wash with cold water for 15 minutes, paying particular attention to area under nails, Immerse in zephiran chloride solution - Inhalation: remove person to fresh air, give artificial respiration as needed and call physician - Ingestion: Do not induce vomiting. Give large amounts of water to victim and call physician

Section 5 FIRE FIGHTING MEASURES

(1.) Extinguisher media: water, foam, or CO2 (2.) Hazardous combustion byproducts: Hydrogen Gas (3.) Recommended fire-fighting procedures: usual procedures__ (4.) Unusual fire explosion hazards: none__

Section 6 ACCIDENTAL RELEASE MEASURES

- Spill or leak procedures: evacuate area, provide plenty of ventilation, absorb liquid and vapors in large volume of water, neutralize with lime - Waste disposal method: dispose of in accordance with Federal, State, and Local regulations - EPA appropriate waste classification: n/a__ (4.) RCRA appropriate characteristic waste: no__ (5.)RCRA appropriate listed waste: no If so, EPA Hazardous Waste # 40 CFR 372 (6.) Procedure for handling empty containers: do not reuse, return to licensed drum reconditioner (7.) Environmental toxicity data: Biodegradable_ (8.) Is material classified under the CLEAN WATER ACT (USA) or appropriate water regulation as a: Toxic pollutant (section 307): No_ Hazardous substance (section 311): YES_ if yes, reportable quantity (R.Q): none___ Oil (section 311): No_ (9.) Is material classified under the CLEAN AIR ACT (USA) or appropriate CLEAN AIR regulation as a: Hazardous air pollutant (section 12): No_ (10.) Comments: None_

Section 7 HANDLING & STORAGE

- Storage: store in cool, dry location away from strong acids, keep container tightly closed, do not store in steel tank to prevent hydrogen gas from forming - Handling: wear chemical resistant gloves and goggles, chemical resistant apron, boots, and face shield - Precautions: avoid eye and skin contact, do not breath vapors or mist, do not wear contact lenses - Unusual Precautions: NONE

Section 8 EXPOSURE CONTROLS/PERSONAL PROTECTION

- Ventilation:_X__ (2.) Local exhaust: X_ (3.) General exhaust: ___ (4.) None required:____ (5.) Personal protective equipment: Ventilation: Corrosion Resistant Fan Respirator type: NIOSH or MSHA Approved Respirator Gloves: Neoprene or PVC (6.) Eye protection: safety goggles___

Section 9 PHYSICAL AND CHEMICAL PROPERTIES

- Appearance: Clear Liquid _____ (2.) Odor: Butyl Odor _______ (3.) Physical State: ____Liquid_____________ - Boiling Point: 152°F______ (5.) Freeze Point: n/a_ (6.) Specific gravity: 1.07__ ) - Water Solubility: Complete_______(8.) Vapor Pressure: n/a________ (9.) Vapor Density: n/a_____________ (10.) Evaporation rate: ≥1 (11.) VOC Less Exempt (Theoretical): 0.0lbs./gal (12.) VOC as Packaged (Theoretical): 0.0lbs./gal *information not provided is either irrelevant or unavailable

Section 10 STABILITY AND REACTIVITY DATA

Reactivity: This product may react with acids Chemical Stability: This chemical is stable under normal ambient temperature and conditions while in storage and being handled Other: - Thermal Stability: STABLE___ If yes, conditions to avoid: irregular conditions_________________________ - Hazardous decomposition products: Hydrogen Gas______________________ - Hazardous polymerization: May occur X___ Will not occur: ______ - Materials to avoid: Strong acids / Alkalis______________________________________________ - Corrosive action on materials: Mild, Dilution increases metal corrosion__________

Section 11 TOXICOLOGICAL INFORMATION

Summary of health effects: Mildly hazardous to all routes of body, seek physician; irritant to skin and eyes (1.) Primary route(s) of entry into body: (2.) Eyes: X__ (3.) Skin:_ X_ (4.) Inhalation: X_ (5.) Ingestion:_X_ (6.) Acute effects: Eyes: tissue irritation, redness, blurred vision Skin: tissue irritation, redness Inhalation: shortness of breath Ingestion: possible nausea, vomiting, gastro intestinal distress *None of the components of this product are recognized as carcinogenic

Section 16 Other Information

SDS was prepared on 6/18/2015. Changes have been made to comply with Appendix D of 29 CFR 1910.1200.

**Sections 12-15 are non-mandatory and have been omitted