- Floor Pads F.Y.I

- Three variables that affect the way a buffer performs:	1-Downward pad pressure

2-Pad contact area

3-Pad speed

| # Pad | Machine RPM | Pad Description |
| --- | --- | --- |
| ## Stripping |  |  |
| ## Black | 150 - 350 | A stripping pad designed to be used for removal of wax, dirt and finish. |
| Brown | 150 - 350 | Excellent for removal of burnished floor finish designed with an open weave construction that enables it to hold more without clogging in heavy build-up area. |
| Diamondback/ Dominator Strip Pad | 150 - 350 | An extremely aggressive strip pad. Works well on problem floors that require a pad with a lot of abrasive to do the stripping. |
| Scrubbing |  |  |
| Blue | 150 - 350 | Medium abrasive pad designed to clean a lightly soiled floor. Can be used to wet scrub. |
| ### Green | 150 -350 | For heavy duty wet scrubbing. Removes heavy dirt and scuffmarks. |
| #### Spray Buffing |  |  |
| #### Red | 150 - 350 | A light abrasive pad used in spray buffing. The fibers are more open to help keep the pad from glazing over. |
| ### White | 150 – 350 | A non-abrasive dry polishing pad, which will level scratches for high luster. |
| Burnishing |  |  |
| ### Beige Rubberized | 1000+ | A totally synthetic, non-aggressive pad, the rubberized coating creates friction needed to repair softer finish. Produces a high gloss shine. |
| ### Porko / Gorilla | 1000+ | Made with a blend of natural fiber and polyester. This pad will give a “wet look” shine. Great with the 1844 Floor Finish |
| ### Cure / Impress | 1000+ | Use to help cure freshly laid finish as soon as it dries. Perfect for overnight recoats before holiday traffic. |
| ### Coconut pad | 1000+ | This is a naturally lubricating fiber pad for UHS burnishing. Suited for frequent burnishing. |

Cleaning:	Polish Pads: Clean often to eliminate build-up that can scratch the floor 				polish. Clean pads by removing them from the machine and shaking them 				in a plastic bag to remove dust & imbedded debris.

Strip or Scrub Pads:	Soak them in a light stripper solution for 5 minutes than brush them with a stiff brush while rinsing them with clear water.

Note: Even though a used pad can be cleaned, it does not mean that it will always produce the same results as a new pad.