TABLE 5-A

STREET DESIGN SPECIFICATIONS

Residential	Max. No. of	Permitted	Required	Min. Pave-		Min.

Street	Dwelling Units/	On-Street	R.O.W.	ment Width	Max. Grad-	Gradient

Classification	Net Acre	Parking	(ft.)	(ft.)	ient (%)	(%)

Marginal Access	To 1.99	None	50	24	6	1.3

Local	2.0-4.50	Both Sides	62	40	6	1.0

Collector	4.50/Greater	Both Sides	66	43	6	1.0

Arterial	Over 250

dwelling units

served	None	70	30	6	1.0

Commercial and			Min.

Industrial	Permitted	Required	Pavement	Max.	Min.

Street	On-Street	R.O.W.	Width	Gradient	Gradient

Classification	Parking	(ft.)	(ft.)	(%)	(%)

Local	None	50	26	10	1.0

Local	One Side	50	34	10	1.0

Local	Both Sides	50	42	10	1.0

Collector	None	60	39	8	1.0

*Parking land width, add ten (10) feet.